# MINUTES

MAPA / MoDOT Quarterly Meeting

November 1, 2022 / 1:00 – 3:00 p.m.

Videoconference / In-person

Agenda Items

- Bit Team meeting update (MoDOT)

- Will rewrite PMTP spec to use RTK accuracy and make effective July 2023 in preparation for 2024 paving season (! 14 – 20 projects).  Also, will change from temperature differential to TSI (thermal segregation index).

- Looking at several solutions for improving longitudinal joint density

  - Notched wedge

  - Remove and replace 6 inches of unconfined edge

  - Void reducing asphalt membrane (VRAM)

- Discussed need to reduce BMD testing frequencies for safety concerns

- Performance Specifications – moving forward (MAPA)

This topic was covered more extensively in the Bit Team meeting.  A MAPA technical team will develop a BMD testing frequency proposal for MoDOT review.

- Future of IC/IR (MAPA)

Another topic discussed more extensively in the Bit Team meeting.  The switch to TSI will better identify thermal streaking.  LIDAR may be specified in the future to reduce risk for QC personnel performing boundary measurements.

- Rejected Bids (MAPA)

MAPA members were concerned about the number of asphalt paving job low bids rejected over the past year.  One example given was a surface level project which was rejected despite having a lower cost/mile relative to a nearby project that had been awarded by the Commission.  The question was raised whether MoDOT could publish engineer’s estimates after a letting, similar to what Kansas does.  Overall, paving prices will continue to increase over the next few years due to trucking shortages and laborers unwilling to work on nights and weekends.

- SL Mix Cold Weather JSP for Low Volume Roads projects (MoDOT)

A JSP was developed to accommodate colder weather paving of routes in the Governor’s Low Volume Roads (LVR) program.  The following four conditions are required for paving to occur when the air temperature is as low as 40° F –

- RAP shall not exceed 20% replacement

- No RAS

- Material transfer vehicle required

- Warm mix technology (chemical or foaming) required

The JSP may be used for other SL projects outside of the LVR program by change order upon request to the RE administering the contract.  MoDOT may eventually move the JSP to the supplemental provisions and perhaps even incorporate into a full standard specification.

- Fuel Adjustment specification change for non-standard items (MoDOT)

Beginning this past July, the fuel index will not be allowed any nonstandard (99 number) paving items, unless specifically allowed in a job JSP.

- New Preliminary Maintenance Specification (104.7.3.1)  (MoDOT)

This specification change reduces the burden on contractors for pre-existing pavement conditions by allowing them to bring obvious needed repairs to the attention of the RE prior to work beginning.  The RE can then determine best how to mitigate, whether with contractor or MoDOT maintenance forces.  Any additional time required for initial repairs can be factored into a contract time extension as deemed appropriate by the RE.

- Design Standards Update (MoDOT)

See handout.

- Sec 806 revised for erosion control items

- SD 606.50 adds note restricting additional payments for cutting over-size holes or placing aggregate for guardrail posts

- SD 616.10 Type 2 portable sign usage to short-term or discontinuous work

- Build America / Buy America (BABA) requirements added to supplemental revisions JSP

- MoDOT working on lump sum temporary traffic control JSP for statewide use.  Also updating 2023 JOCs.

- Other items

- MAPA conference January 9th - 11th

- MAPA training in February 2023 will add a QC/QA module that is no longer in the certification course since it has been taught at State Tech in Linn.

- UPDATE – new MODOT Construction and Materials Engineer is Brandi Baldwin.

Next Meeting Date:  January 31, 2023