30 degee safety edge requirment.  Industry requesting clarification when safety edge should be used and there seems to be inconsistent usage.  Federally mandated safety treatment intended for shoulder-less pavements where there is space for a safety slope.  The intent is that if aggregate backfill erodes away; a safety edge is there to help mitigate accidents.  The Safety Edge is:

    - Required any time for shoulder-less pavement or the shoulder is 4 ft wide or less provided the roadway has space to support the safety edge.

    - Intended for routes with aggregate shoulders that have edge ruts/etc., but not to narrow the road surface.

    - Generally, not done for lettered routes as they don’t have the width to support the safety edge

    - Missouri took it a little farther to include paved shoulders if they are 4 feet wide or less

    - Typically requires 30° edge on the pavement that is backfilled with aggregate

    - NE/CD use a flatter safety edge with no backfill but use a 3 or 4 to 1 slope on the asphalt

    - Asphalt Industry to follow up with other states on what they call their equivalent treatment, if they call it something else than “Safety Edge”.  Concerns of liability of calling it “Safety Edge”.